IIT Jammu Summer School 2026

Tittle

Project 10: Full IoT Home Automation Hub

Submitted By

Name: RUBADHARSHINI S

Department: Electronics and Communication Engineering

College: Arjun College of Technology

Course: IoT & Drones

Objective

The objective of this project is to develop a smart home automation system using the ESP32. The system monitors environmental conditions such as temperature, humidity, gas leakage, light intensity, and motion. It automatically controls electrical devices based on sensor readings and provides remote monitoring through a web dashboard and MQTT communication.

Components Used

| S.No | Component | Quantity |
| --- | --- | --- |
| 1 | ESP32 Dev Board | 1 |
| 2 | DHT11 Temperature & Humidity Sensor | 1 |
| 3 | MQ-2 Gas Sensor | 1 |
| 4 | PIR Motion Sensor | 1 |
| 5 | LDR Sensor | 1 |
| 6 | 2-Channel Relay Module | 1 |
| 7 | OLED Display (SSD1306 I2C) | 1 |
| 8 | Active Buzzer | 1 |
| 9 | Red LED | 1 |
| 10 | Green LED | 1 |
| 11 | Yellow LED | 1 |
| 12 | Push Buttons | 2 |
| 13 | Breadboard | 1 |
| 14 | Jumper Wires | As Required |

Circuit Connections

DHT11

- VCC → 3.3V

- GND → GND

- DATA → GPIO4

MQ-2 Gas Sensor

- VCC → 5V

- GND → GND

- AOUT → GPIO35

PIR Sensor

- VCC → 5V

- GND → GND

- OUT → GPIO13

LDR

- One terminal → 3.3V

- Other terminal → GPIO34

- 10kΩ resistor → GND

Relay Module

- Relay 1 → GPIO26

- Relay 2 → GPIO27

- VCC → 5V

- GND → GND

OLED Display

- SDA → GPIO21

- SCL → GPIO22

- VCC → 3.3V

- GND → GND

LEDs

- Red LED → GPIO25

- Green LED → GPIO2

- Yellow LED → GPIO33

Buzzer

- Positive → GPIO14

- Negative → GND

Push Buttons

- Fan Override → GPIO0

- Light Override → GPIO32

Working Principle

The ESP32 continuously reads the values from the DHT11, MQ-2, PIR, and LDR sensors. Based on these readings, the system automatically controls the fan and light using relay modules.

If the room temperature rises above 32°C, the fan relay turns ON automatically. The fan turns OFF when the temperature drops below 28°C.

The light relay turns ON only when the room is dark and motion is detected by the PIR sensor. It turns OFF when daylight is available or when no motion is detected for three minutes.

If the MQ-2 gas sensor detects a dangerous gas level above the threshold, the buzzer and red LED are activated, all relays are turned OFF for safety, and an alert message is published using MQTT.

The OLED display continuously shows sensor values and relay status. The ESP32 also hosts a web dashboard where users can monitor sensor readings and manually control the relays.

Program

#include <WiFi.h>

#include <WebServer.h>

#include <PubSubClient.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <DHT.h>

const char* ssid = "ECE-LAB-01 1161";

const char* password = "123456789";

const char* mqtt_server = "broker.hivemq.com";

WiFiClient espClient;

PubSubClient mqttClient(espClient);

String mqttTopic = "iitjammu/RUBADHARSHINI/home";

// DHT11

#define DHTPIN 4

#define DHTTYPE DHT11

// Sensors

#define MQ2_PIN 35

#define PIR_PIN 13

#define LDR_PIN 34

// Relay

#define FAN_RELAY 26

#define LIGHT_RELAY 27

// Alert

#define BUZZER_PIN 14

#define RED_LED 25

#define GREEN_LED 2

// OLED

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_SDA 21

#define OLED_SCL 22

DHT dht(DHTPIN, DHTTYPE);

Adafruit_SSD1306 display(

SCREEN_WIDTH,

SCREEN_HEIGHT,

&Wire,

-1

);

WebServer server(80);

float temperature = 0;

float humidity = 0;

int gasValue = 0;

int gasPercent = 0;

int ldrValue = 0;

int lightPercent = 0;

int pirState = 0;

bool fanState = false;

bool lightState = false;

bool gasAlert = false;

unsigned long lastSensorRead = 0;

unsigned long lastMQTTPublish = 0;

unsigned long lastOLEDUpdate = 0;

int oledScreen = 0;

// Light motion timer

unsigned long lastMotionTime = 0;

void setup()

{

Serial.begin(115200);

// Pin modes

pinMode(MQ2_PIN, INPUT);

pinMode(PIR_PIN, INPUT);

pinMode(LDR_PIN, INPUT);

pinMode(FAN_RELAY, OUTPUT);

pinMode(LIGHT_RELAY, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(RED_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

// Relay OFF initially

digitalWrite(FAN_RELAY, HIGH);

digitalWrite(LIGHT_RELAY, HIGH);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

// DHT start

dht.begin();

// OLED start

Wire.begin(OLED_SDA, OLED_SCL);

if (!display.begin(

SSD1306_SWITCHCAPVCC,

0x3C

))

{

Serial.println("OLED not found");

while (true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(0, 0);

display.println("IoT HOME HUB");

display.println("Starting...");

display.display();

delay(2000);

// Wi-Fi

connectWiFi();

// MQTT

mqttClient.setServer(

mqtt_server,

1883

);

// Web routes

server.on("/", handleRoot);

server.on(

"/fan/on",

[]()

{

fanState = true;

digitalWrite(FAN_RELAY, LOW);

server.send(200, "text/plain", "Fan ON");

}

);

server.on(

"/fan/off",

[]()

{

fanState = false;

digitalWrite(FAN_RELAY, HIGH);

server.send(200, "text/plain", "Fan OFF");

}

);

server.on(

"/light/on",

[]()

{

lightState = true;

digitalWrite(LIGHT_RELAY, LOW);

server.send(200, "text/plain", "Light ON");

}

);

server.on(

"/light/off",

[]()

{

lightState = false;

digitalWrite(LIGHT_RELAY, HIGH);

server.send(200, "text/plain", "Light OFF");

}

);

server.begin();

Serial.println("Web Server Started");

}

void loop()

{

server.handleClient();

if (!mqttClient.connected())

{

reconnectMQTT();

}

mqttClient.loop();

// Read sensors every 5 seconds

if (millis() - lastSensorRead >= 5000)

{

readSensors();

applyAutomationRules();

updateOLED();

lastSensorRead = millis();

}

// MQTT every 30 seconds

if (millis() - lastMQTTPublish >= 30000)

{

publishMQTT();

lastMQTTPublish = millis();

}

}

void connectWiFi()

{

Serial.print("Connecting WiFi");

WiFi.begin(

ssid,

password

);

while (WiFi.status() != WL_CONNECTED)

{

delay(500);

Serial.print(".");

}

Serial.println();

Serial.println("WiFi Connected");

Serial.print("IP Address: ");

Serial.println(

WiFi.localIP()

);

}

void readSensors()

{

// DHT11

float newTemperature = dht.readTemperature();

float newHumidity = dht.readHumidity();

if (!isnan(newTemperature))

{

temperature = newTemperature;

if (!isnan(newHumidity))

{

humidity = newHumidity;

}

// MQ2

gasValue = analogRead(MQ2_PIN);

gasPercent = map(

gasValue,

0,

4095,

0,

100

);

gasPercent = constrain(

gasPercent,

0,

100

);

// PIR

pirState = digitalRead(PIR_PIN);

// LDR

ldrValue = analogRead(LDR_PIN);

lightPercent = map(

ldrValue,

0,

4095,

0,

100

);

lightPercent = constrain(

lightPercent,

0,

100

);

if (pirState == HIGH)

{

lastMotionTime = millis();

}

Serial.println("--------------------");

Serial.print("Temperature: ");

Serial.println(temperature);

Serial.print("Humidity: ");

Serial.println(humidity);

Serial.print("Gas: ");

Serial.println(gasPercent);

Serial.print("PIR: ");

Serial.println(pirState);

Serial.print("Light: ");

Serial.println(lightPercent);

}

void applyAutomationRules()

{

if (gasPercent > 60)

{

gasAlert = true;

// Safety shutdown

fanState = false;

lightState = false;

digitalWrite(

FAN_RELAY,

HIGH

);

digitalWrite(

LIGHT_RELAY,

HIGH

);

digitalWrite(

BUZZER_PIN,

HIGH

);

digitalWrite(

RED_LED,

HIGH

);

digitalWrite(

GREEN_LED,

LOW

);

return;

}

else

{

gasAlert = false;

digitalWrite(

BUZZER_PIN,

LOW

);

digitalWrite(

RED_LED,

LOW

);

digitalWrite(

GREEN_LED,

HIGH

);

}

// Temperature above 32°C

if (!fanState && temperature > 32)

{

fanState = true;

digitalWrite(

FAN_RELAY,

LOW

);

Serial.println("Fan AUTO ON");

}

// Temperature below 28°C

if (fanState && temperature < 28)

{

fanState = false;

digitalWrite(

FAN_RELAY,

HIGH

);

Serial.println("Fan AUTO OFF");

}

// Dark + Motion

if (

lightPercent < 20 &&

pirState == HIGH

)

{

lightState = true;

digitalWrite(

LIGHT_RELAY,

LOW

);

Serial.println("Light AUTO ON");

}

// Daylight

if (lightPercent > 60)

{

lightState = false;

digitalWrite(

LIGHT_RELAY,

HIGH

);

Serial.println("Light OFF - Daylight");

}

// No motion for 3 minutes

if (

millis() - lastMotionTime >= 180000

)

{

lightState = false;

digitalWrite(

LIGHT_RELAY,

HIGH

);

Serial.println(

"Light OFF - No Motion"

);

}

}

void reconnectMQTT()

{

while (!mqttClient.connected())

{

Serial.print("Connecting MQTT...");

String clientID = "ESP32HomeHub";

clientID += String(

random(0xffff),

HEX

);

if (

mqttClient.connect(

clientID.c_str()

)

)

{

Serial.println(

"MQTT Connected"

);

}

else

{

Serial.print(

"Failed, rc="

);

Serial.println(

mqttClient.state()

);

delay(2000);

}

}

}

void publishMQTT()

{

String json = "{";

json += "\"temp\":";

json += String(temperature, 1);

json += ",\"humidity\":";

json += String(humidity, 1);

json += ",\"gas\":";

json += String(gasPercent);

json += ",\"pir\":";

json += String(pirState);

json += ",\"light\":";

json += String(lightPercent);

json += ",\"fan\":";

json += String(fanState ? 1 : 0);

json += ",\"light_relay\":";

json += String(lightState ? 1 : 0);

json += ",\"alert\":";

json += String(gasAlert ? 1 : 0);

json += "}";

mqttClient.publish(

mqttTopic.c_str(),

json.c_str()

);

Serial.println(

"MQTT Published:"

);

Serial.println(json);

}

void updateOLED()

{

if (

millis() - lastOLEDUpdate >= 5000

)

{

oledScreen++;

if (oledScreen > 2)

{

oledScreen = 0;

}

lastOLEDUpdate = millis();

}

display.clearDisplay();

display.setCursor(0, 0);

// Screen 1

if (oledScreen == 0)

{

display.println(

"ENVIRONMENT"

);

display.println();

display.print(

"Temp: "

);

display.print(

temperature

);

display.println(

" C"

);

display.print(

"Humidity: "

);

display.print(

humidity

);

display.println(

" %"

);

}

// Screen 2

else if (oledScreen == 1)

{

display.println(

"SENSORS"

);

display.println();

display.print(

"Gas: "

);

display.print(

gasPercent

);

display.println(

" %"

);

display.print(

"PIR: "

);

display.println(

pirState ? "MOTION" : "NO MOTION"

);

display.print(

"LDR: "

);

display.print(

lightPercent

);

display.println(

" %"

);

}

// Screen 3

else

{

display.println(

"SYSTEM STATUS"

);

display.println();

display.print(

"Fan: "

);

display.println(

fanState ? "ON" : "OFF"

);

display.print(

"Light: "

);

display.println(

lightState ? "ON" : "OFF"

);

display.print(

"Alert: "

);

display.println(

gasAlert ? "YES" : "NO"

);

}

display.display();

}

void handleRoot()

{

String html = "";

html += "<!DOCTYPE html>";

html += "<html lang='en'>";

html += "<head>";

html += "<meta charset='UTF-8'>";

html += "<meta name='viewport' content='width=device-width, initial-scale=1'>";

html += "<meta http-equiv='refresh' content='5'>";

html += "<title>ESP32 IoT Home Hub</title>";

html += "<style>";

html += "*{box-sizing:border-box;margin:0;padding:0;}";

html += "body{";

html += "font-family:'Segoe UI',Roboto,Arial,sans-serif;";

html += "background:linear-gradient(135deg,#1f2937,#0f172a);";

html += "min-height:100vh;";

html += "padding:24px 14px 60px;";

html += "color:#e5e7eb;";

html += "}";

html += ".wrap{max-width:760px;margin:0 auto;}";

html += "header{text-align:center;margin-bottom:24px;}";

html += "header h1{";

html += "font-size:1.6rem;";

html += "font-weight:700;";

html += "background:linear-gradient(90deg,#38bdf8,#818cf8);";

html += "-webkit-background-clip:text;";

html += "background-clip:text;";

html += "color:transparent;";

html += "}";

html += "header p{color:#94a3b8;font-size:0.85rem;margin-top:4px;}";

html += ".alert-banner{";

html += "text-align:center;";

html += "padding:12px;";

html += "border-radius:12px;";

html += "font-weight:700;";

html += "letter-spacing:0.5px;";

html += "margin-bottom:20px;";

html += "font-size:1rem;";

html += "}";

html += ".alert-safe{background:rgba(34,197,94,0.15);color:#4ade80;border:1px solid rgba(34,197,94,0.4);}";

html += ".alert-danger{background:rgba(239,68,68,0.18);color:#f87171;border:1px solid rgba(239,68,68,0.5);";

html += "animation:pulse 1.2s infinite;}";

html += "@keyframes pulse{0%{opacity:1;}50%{opacity:0.55;}100%{opacity:1;}}";

html += ".grid{";

html += "display:grid;";

html += "grid-template-columns:repeat(auto-fit,minmax(220px,1fr));";

html += "gap:16px;";

html += "margin-bottom:16px;";

html += "}";

html += ".card{";

html += "background:rgba(255,255,255,0.06);";

html += "border:1px solid rgba(255,255,255,0.08);";

html += "backdrop-filter:blur(6px);";

html += "border-radius:16px;";

html += "padding:18px 20px;";

html += "box-shadow:0 4px 14px rgba(0,0,0,0.25);";

html += "}";

html += ".card h2{";

html += "font-size:0.85rem;";

html += "text-transform:uppercase;";

html += "letter-spacing:1px;";

html += "color:#93c5fd;";

html += "margin-bottom:12px;";

html += "display:flex;align-items:center;gap:8px;";

html += "}";

html += ".row{";

html += "display:flex;";

html += "justify-content:space-between;";

html += "align-items:center;";

html += "padding:6px 0;";

html += "border-bottom:1px dashed rgba(255,255,255,0.08);";

html += "}";

html += ".row:last-child{border-bottom:none;}";

html += ".row .label{color:#cbd5e1;font-size:0.9rem;}";

html += ".row .value{font-weight:700;font-size:1.05rem;}";

html += ".pill{";

html += "display:inline-block;";

html += "padding:3px 12px;";

html += "border-radius:999px;";

html += "font-size:0.8rem;";

html += "font-weight:700;";

html += "}";

html += ".pill-on{background:rgba(34,197,94,0.18);color:#4ade80;}";

html += ".pill-off{background:rgba(148,163,184,0.18);color:#94a3b8;}";

html += ".pill-motion{background:rgba(56,189,248,0.18);color:#38bdf8;}";

html += ".pill-nomotion{background:rgba(148,163,184,0.18);color:#94a3b8;}";

html += ".btn-row{display:flex;gap:10px;margin-top:14px;}";

html += ".btn{";

html += "flex:1;";

html += "text-align:center;";

html += "padding:10px 8px;";

html += "border-radius:10px;";

html += "text-decoration:none;";

html += "font-weight:600;";

html += "font-size:0.85rem;";

html += "transition:transform 0.15s ease;";

html += "}";

html += ".btn:active{transform:scale(0.96);}";

html += ".btn-on{background:#22c55e;color:#052e14;}";

html += ".btn-off{background:#334155;color:#e2e8f0;}";

html += "footer{";

html += "text-align:center;";

html += "color:#64748b;";

html += "font-size:0.75rem;";

html += "margin-top:24px;";

html += "}";

html += "</style>";

html += "</head>";

html += "<body>";

html += "<div class='wrap'>";

html += "<header>";

html += "<h1>&#127968; ESP32 IoT Home Automation Hub</h1>";

html += "<p>Live sensor data &bull; auto-refreshes every 5 seconds</p>";

html += "</header>";

if (gasAlert)

{

html += "<div class='alert-banner alert-danger'>&#9888; GAS ALERT - SAFETY SHUTDOWN ACTIVE</div>";

}

else

{

html += "<div class='alert-banner alert-safe'>&#10003; ALL SYSTEMS SAFE</div>";

}

html += "<div class='grid'>";

// Environment card

html += "<div class='card'>";

html += "<h2>&#127777; Environment</h2>";

html += "<div class='row'><span class='label'>Temperature</span>";

html += "<span class='value'>";

html += String(temperature);

html += " &deg;C</span></div>";

html += "<div class='row'><span class='label'>Humidity</span>";

html += "<span class='value'>";

html += String(humidity);

html += " %</span></div>";

html += "</div>";

// Sensors card

html += "<div class='card'>";

html += "<h2>&#128269; Sensors</h2>";

html += "<div class='row'><span class='label'>Gas Level</span>";

html += "<span class='value'>";

html += String(gasPercent);

html += " %</span></div>";

html += "<div class='row'><span class='label'>Motion (PIR)</span>";

if (pirState)

{

html += "<span class='pill pill-motion'>MOTION</span>";

}

else

{

html += "<span class='pill pill-nomotion'>NO MOTION</span>";

}

html += "</div>";

html += "<div class='row'><span class='label'>Light Level</span>";

html += "<span class='value'>";

html += String(lightPercent);

html += " %</span></div>";

html += "</div>";

html += "</div>"; // end grid

html += "<div class='grid'>";

// Fan control card

html += "<div class='card'>";

html += "<h2>&#128168; Fan Relay</h2>";

html += "<div class='row'><span class='label'>Status</span>";

if (fanState)

{

html += "<span class='pill pill-on'>ON</span>";

}

else

{

html += "<span class='pill pill-off'>OFF</span>";

}

html += "</div>";

html += "<div class='btn-row'>";

html += "<a class='btn btn-on' href='/fan/on'>Turn ON</a>";

html += "<a class='btn btn-off' href='/fan/off'>Turn OFF</a>";

html += "</div>";

html += "</div>";

// Light control card

html += "<div class='card'>";

html += "<h2>&#128161; Light Relay</h2>";

html += "<div class='row'><span class='label'>Status</span>";

if (lightState)

{

html += "<span class='pill pill-on'>ON</span>";

}

else

{

html += "<span class='pill pill-off'>OFF</span>";

}

html += "</div>";

html += "<div class='btn-row'>";

html += "<a class='btn btn-on' href='/light/on'>Turn ON</a>";

html += "<a class='btn btn-off' href='/light/off'>Turn OFF</a>";

html += "</div>";

html += "</div>";

html += "</div>"; // end grid

html += "<footer>ESP32 Home Automation Hub &bull; IIT Jammu Summer School 2026</footer>";

html += "</div>"; // end wrap

html += "</body>";

html += "</html>";

server.send(

200,

"text/html",

html

);

}

Automation Rules

Fan

- ON when Temperature > 32°C

- OFF when Temperature < 28°C

Light

- ON when LDR detects darkness and PIR detects motion

- OFF when daylight is detected or no motion is detected for 3 minutes

Gas Alert

- Gas Level > 60%

- Red LED ON

- Buzzer ON

- All Relays OFF

- MQTT Alert Published

Features

- Real-time temperature monitoring

- Humidity monitoring

- Gas leakage detection

- Motion detection

- Automatic fan control

- Automatic light control

- OLED display

- Web dashboard

- MQTT communication

- Manual relay control

- Gas safety shutdown

- System uptime monitoring

Expected Output

- OLED displays sensor values.

- Fan turns ON automatically at high temperature.

- Light turns ON in darkness when motion is detected.

- Gas leakage activates buzzer and red LED.

- Web dashboard displays live sensor readings.

- MQTT publishes sensor data every 30 seconds.

- Manual buttons override automatic control.

Serial Monitor Output

Web Dashboard Image

Applications

- Smart Home Automation

- Hostel Room Monitoring

- Energy Saving Systems

- Gas Leakage Detection

Advantages

- Reduces power consumption

- Improves safety

- Automatic device control

- Remote monitoring

Future Scope

- Mobile App Control

- Voice Assistant Integration

- Cloud Data Storage

- Mobile Notifications

Conclusion

The Full IoT Home Automation Hub successfully integrates multiple sensors with the ESP32 to create a smart and automated home monitoring system. It provides real-time monitoring, automatic appliance control, gas safety alerts, and remote access through a web dashboard and MQTT. This project demonstrates practical IoT concepts and can be extended for real-world smart home applications.